New Measurements of Gamma-rays from Thermal Neutron Capture on Manganese

Howe, Alex1, Jandel, Marian1, Fernandez, Daniel1, Fishbein, Aaron1, McCutchan, Elizabeth2, Ota, Shuya2, Valenta, Stanislav3

1Department of Physics and Applied Physics, University of Massachusetts Lowell, United States

2Brookhaven National Laboratory, United States

3Faculty of Mathematics and Physics, Charles University, Czech Republic

Current data on the production cross sections of thermal neutron capture γ-rays from the reaction 55Mn(n, γ)56Mn needs improvement to resolve discrepancies between libraries [1-3] and to revisit experimental methods with high-resolution γ-ray detectors. Neutron capture on manganese is important for applications of neutron activation, such as planetary γ-ray spectroscopy and assaying unknown isotopes. We carried out new measurements of capture γ-rays from the 55Mn(n, γ)56Mn reaction at the University of Massachusetts Lowell Research Reactor [4]. Measurements were performed by irradiating 55Mn and MnCl2 samples in a 1-inch collimated thermal neutron beam. Gamma-rays were measured with two high-purity germanium (HPGe) detectors with 30% relative efficiency. One detector was placed inside active Compton shielding from BGO and NaI scintillators. Detection efficiency was calibrated between γ-ray energies of 0.1 and 10 MeV using 152Eu and 35Cl(n, γ)36Cl [5]. The 35Cl(n, γ)36Cl reaction was also used to normalize production of capture γ-rays from 55Mn. We will present results of γ-ray intensities and production cross sections between 2 and 8 MeV. The γ-ray spectra from experiment will be compared to GEANT4 simulations.

References:

[1] M. Mauborgne et. al. (2020) EPJ WoC, 239, 20007 (ND2019).

[2] M. J. Martin (2013) NDS 114, 1497.

[3] R. B. Firestone (2007) IAEA, 96-97.

[4] M. Jandel et. al. (2025) Nucl. Phys. A. (1060), 123116.

[5] Y. Ma et. al. (2023) J. Radioanal. Nucl. Chem. 332, 4009-4018.