Current Status on the INL HERMES-400 Gamma-Ray Microcalorimeter

Abel, E. Paige1, Williams, Ammon1, Bucher, Brian1, Seabury, Edward1, Davis, Victoria1, Schreiber, Katherine2, Croce, Mark2, Becker, Daniel4, Matthew Carpenter2, Mark Keller4, Joel Ullom4, Dan Swetz4, Daniel McNeel 2, Daniel Schmidt4, Johnathon Gard3, Douglas Bennett4, John Mates4, Jason Nobles3

- Idaho National Laboratory, Idaho Falls, ID, United States

- Los Alamos National Laboratory, Los Alamos, NM, United States

- University of Colorado Boulder, Boulder, CO, United States

- National Institute of Standards and Technology, Boulder, CO, United States

Microcalorimeter gamma-ray spectrometers are next-generation gamma detectors that can provide significantly higher energy resolution than traditional gamma detectors. Microcalorimeters are anticipated to achieve 5 to 10 times higher energy resolution than many commercial high purity germanium (HPGe) detectors. For example, some current microcalorimeters can achieve a full width half maximum (FWHM) of less than 100 eV for 100 keV peaks compared to 550 eV FWHM reported for commercial HPGe detectors [1, 2]. Higher peak resolution could improve peak separation for spectra with a high density of gamma peaks. Additionally, improved resolution could lead to higher precision measurements for activity quantification or isotopic ratio measurements [3]. As microcalorimetry resolution continues to improve, this nondestructive assay technique could be used for precision measurements rather than more time-consuming destructive assay methods that require sample dissolution and significant hands-on work [4].

The current status of the High Efficiency and Resolution Microcalorimeter Spectrometer with a 400-pixel TES array (HERMES-400) located at Idaho National Laboratory will be reported on. Additionally, measurements of spent fuel from a pressurized water reactor and irradiated TRISO fuel will be presented, demonstrating up to 7 times improved energy resolution compared to HPGe spectra. Finally, several detector hardware improvements to enhance energy resolution will be discussed.

References:

[1]	U-LEGe Ultra-Low Energy Germanium Detectors. (2023). https://www.mirion.com/products/technologies/spectroscopy-scientific-analysis/gamma-spectroscopy/detectors/hpge-detectors-accessories/u-lege-ultra-low-energy-germanium-detectors.

[2]	J. N. Ullom, D. A. Bennett, J. N. Ullom, and D. A. Bennett, "Review of superconducting transition-edge sensors for x-ray and gamma-ray spectroscopy*," Superconductor Science and Technology, vol. 28, no. 8, 2015-07-13, doi: 10.1088/0953-2048/28/8/084003.

[3]	D. Becker et al., "Advances in Analysis of Microcalorimeter Gamma-Ray Spectra," IEEE Transactions on Nuclear Science, vol. 66, no. 12, 2019, doi: 10.1109/TNS.2019.2953650.

[4]	M. Croce, D. Becker, K. E. Koehler, and J. Ullom, "Improved Nondestructive Isotopic Analysis with Practical Microcalorimeter Gamma Spectrometers," 2021/04/07, doi: 10.48550/arXiv.2104.03376.